Ex.No:1: Simulation of FDMA in MATLAB  Date:21-7-26

Aim: To simulate a Frequency Division Multiple Access (FDMA) system using MATLAB and analyze its communication performance in terms of channel bandwidth, data rate, and signal-to-noise ratio (SNR).

# Objective 1:

To	simulate	an	FDMA	system	by	allocating	individual frequency channels to multiple users

and	determine	the	channel	bandwidth	assigned	to	each user.Output Parameter: Channel Bandwidth (kHz)

# Objective 2:

To calculate the data rate achieved by each user in the FDMA system based on the allocated channel bandwidth.

Output Parameter: Data Rate (kbps)

# Objective 3:

To evaluate the communication quality of the FDMA system by calculating	the	Signal-to-Noise	Ratio	(SNR)	for	different users.Output Parameter: Signal-to-Noise Ratio (SNR) (dB) Procedure

- Open MATLAB and create a new script file.

- Define the total available bandwidth, number of FDMA users, carrier frequencies, signal

power, and noise power.

- Allocate	a	unique	frequency	channel	to	each	user	and calculate the channel bandwidth.

- Compute the data rate for each user based on the allocated bandwidth and modulation

scheme.

- Calculate the Signal-to-Noise Ratio (SNR) using the signal and noise power values.

- Plot the transmitted signals of all FDMA users using different carrier frequencies.

- Display the calculated channel bandwidth, data rate, and SNR in the MATLAB

Command Window.

- Analyze the results to verify independent channel allocation and evaluate the overall

performance of the FDMA System

## Program (Matlab Code):

clc; clear; close all; fs=1000; t=0:1/fs:1; TBW=600; Users=3;

CBW=TBW/Users; fc=[100 300 500];

m=sin(2*pi*10*t); s1=m.*cos(2*pi*fc(1)*t); s2=m.*cos(2*pi*fc(2)*t); s3=m.*cos(2*pi*fc(3)*t); subplot(2,1,1) plot(t,s1,'r',t,s2,'g',t,s3,'b'),grid on

title('FDMA Channel Allocation'),xlabel('Time (s)'),ylabel('Amplitude')

subplot(2,1,2) bar(1:Users,CBW*ones(1,Users)),grid on

xlabel('Users'),ylabel('Channel Bandwidth (kHz)'),title('Channel Bandwidth per User')

## Output: Objective 1:

## Objective 2: Matlab Code and Output

clc; clear; close all;

BW = 200; % Channel Bandwidth (kHz) Users = 3;

M = 2; % BPSK Modulation

DataRate = BW*log2(M); % Data Rate (kbps) Rate = DataRate*ones(1,Users);

disp(' User Data Rate (kbps)') disp([(1:Users)' Rate']) figure bar(1:Users,Rate) xlabel('Users')

ylabel('Data Rate (kbps)')

title('FDMA Data Rate for Each User') grid on

## Output: Objective 2:

Objective 3: Matlab Code and Ootput clc; clear; close all;

Ps = 1; % Signal Power (W)

Pn = [0.1 0.05 0.01]; % Noise Power (W)

Users = 1:3;

SNR = 10*log10(Ps./Pn); % SNR in dB disp('User Noise Power(W) SNR(dB)') disp([Users' Pn' SNR'])

figure bar(Users,SNR) xlabel('Users') ylabel('SNR (dB)')

title('Signal-to-Noise Ratio of FDMA Users') grid on

## Output: Objective 3:

Result:

- The FDMA system was successfully simulated by allocating separate frequency channels

to three users, and the channel bandwidth assigned to each user was determined.

- The data rate achieved by each user was successfully calculated based on the allocated

channel bandwidth, demonstrating equal transmission capacity for all users.

- The Signal-to-Noise Ratio (SNR) for different users was successfully evaluated,

showing that higher SNR values provide better communication quality and reliable data

transmission.